Response Letter

Manuscript ID: TGCN-RP-A1-26-0009;

Manuscript Title: Energy Efficiency Optimization of Passive RIS-Aided ISAC Systems with the Unknown Target: A Deep Learning Method.

Reviewer: 1

Comment 1: The literature review is not satisfied. The corresponding context just listed several different studies, but the pros and cons of these studies have not been illustrated. （文献综述未能令人满意。相关部分仅列举了若干不同的研究，但并未阐明这些研究各自的优缺点。）

Thank you for this valuable comment. We agree that the original literature review mainly summarized individual studies without sufficiently discussing their advantages and limitations. In the revised manuscript, we have reorganized the related work into model-driven optimization methods, general learning-based wireless optimization methods, and learning-based RIS-assisted ISAC methods. For each category, we now discuss its main advantages, required assumptions, and limitations. In particular, we clarify that conventional optimization methods provide interpretable and constraint-aware solutions but require accurate CSI and iterative computation, whereas existing learning-based methods reduce online complexity but generally assume that target-related CSI is already available. These discussions have been added to Section I to better motivate the proposed two-stage framework.

Introduction

These model-driven methods provide interpretable solutions and explicit constraint handling under accurate CSI. However, their iterative procedures incur considerable online complexity and are sensitive to cascaded CSI errors, especially in large-scale and multi-user RIS systems.

Learning-based methods reduce online computation by learning the mapping from CSI to system configurations. However, unlike the proposed framework, existing optimization-based methods [9]–[12] generally rely on iterative optimization with available CSI, while learning-based RIS-ISAC methods [20],[21],[25],[26] typically optimize the system configuration after the target-related CSI has been acquired.

Comment 2: The background and motivation require further improvement. The limitations of prior work and the specific research gap addressed in this study should be clearly articulated. （研究背景与动机尚需进一步完善。应明确阐述既往研究的局限性以及本研究所填补的具体研究空白。）

We agree that the original motivation was not sufficiently explicit. The revised Introduction now clarifies three key gaps: most existing methods require perfect or explicitly estimated CSI, localization and beamforming optimization are generally treated separately, and sensing-oriented energy efficiency under an initially unknown passive target has received limited attention. These gaps are now directly connected to the proposed two-stage framework.

Consequently, a gap remains between target-information acquisition and system-configuration optimization. For an unknown passive target, the BS cannot directly obtain the target-related CSI through pilot transmission or feedback, whereas the RIS phase shifts and BS beamforming vectors strongly depend on such information[27]. Moreover, sensing EE optimization that explicitly utilizes the estimated target location remains insufficiently investigated. To bridge this gap, we propose a two-stage DL-based framework that first estimates the target location and constructs position-aware CSI, and then jointly optimizes the RIS phase shifts and BS beamforming vectors to maximize the sensing EE.

Comment 3: A comparison with benchmark studies and closely related works is missing. Such comparisons are necessary to properly position the contribution and clearly demonstrate its novelty. （文中缺少与基准研究及密切相关工作的对比。此类对比对于准确界定本研究的贡献并清晰展示其创新性至关重要。）【！！！慎重添加表格，还是用话来描述比较好！！！】

Thank you for this important comment. We agree that the distinction between the proposed method and closely related studies was not sufficiently clear in the original manuscript. In the revised Introduction, we have added a focused textual comparison with the optimization-based studies [9]–[12] and the learning-based RIS-assisted ISAC methods [18]–[21]. We now clarify that conventional methods generally rely on iterative optimization with available CSI, while existing learning-based methods typically optimize the system configuration after the target-related CSI has been acquired. In contrast, the proposed framework explicitly connects unknown-target localization, position-aware CSI construction, and sensing EE optimization. We have also expanded the simulation section to explain the selection and optimization mechanisms of the BCD, SLBM, and PF benchmarks.

To evaluate the proposed method, we consider three representative benchmarks. The BCD-based method alternately optimizes the BS beamforming vectors and RIS phase shifts. The SLBM-based method [37] replaces the original non-convex objective with a sequence of tractable lower-bound approximations. The PF-based method [38]determines the RIS phases according to a predefined phase-alignment rule. These benchmarks represent iterative joint optimization, successive approximation, and low-complexity heuristic phase control, respectively.

Comment 4: Please emphasis discussion on feasible region.

(增加可行域的实验、请重点讨论可行域。)

Thank you for this valuable suggestion. We have added an explicit definition and discussion of the feasible region after problem (16) in the revised manuscript. The revised discussion explains that the feasible region is non-convex due to the unit-modulus RIS constraints, the coupling among multi-user SINR constraints, and the joint dependence of the sensing SNR on the RIS phase shifts and beamforming vectors. Furthermore, we analyze how the feasible region is affected by the transmit-power budget, communication SINR requirements, sensing-SNR threshold, and CSI estimation errors.

In addition, we have added a constraint satisfaction analysis in the simulation section. Specifically, the satisfaction rates of the transmit-power, communication-SINR, and sensing-SNR constraints in (16c)–(16e), as well as the joint feasibility rate, are evaluated on the independent test set.

Define the feasible region of problem (16) as

F={(θ,{wk})∣(16b)-(16e)}.

The feasible region is non-convex because of the unit-modulus RIS constraint and the coupled sensing-SNR constraint. Moreover, the multi-user SINR constraints couple the beamforming vectors through inter-user interference. Increasing the minimum communication SINR requirements {γk}or the sensing-SNR threshold τrreduces the feasible region, whereas increasing the transmit-power budget generally enlarges it. When the communication and sensing requirements are simultaneously stringent, the feasible region may become empty. Furthermore, under target-location and CSI estimation errors, a solution that is feasible for the reconstructed channel may violate the constraints under the actual channel.

Comment 5: Numerous studies are available on same topic. The authors should review these works carefully and clearly highlight the distinct contributions of the present paper.   （关于同一主题已有大量研究。作者应仔细审视这些文献，并明确阐述本文的独特贡献。）【这个全文总结性的，等全文改差不多了再次修改，目前的修改先不在文章中体现】

Thank you for this valuable suggestion. We agree that the distinct contributions of this work were not sufficiently highlighted in the original manuscript. We have carefully re-examined the closely related studies and revised the Introduction and contribution statements. In the revised manuscript, commonly used training techniques are no longer considered as contributions. Instead, the contributions are clarified from the perspectives of the problem formulation, framework design, and optimization objective. The differences from existing RIS-assisted ISAC studies are further discussed in the revised Introduction.

The main contributions are summarized as follows:

- We investigate sensing energy-efficiency optimization in a passive RIS-assisted multi-user ISAC system with an initially unknown sensing target. To address the unavailability of target-related CSI, a location-aware optimization framework is established, where target localization is performed before RIS phase and beamforming optimization.

- A multi-task localization network is developed to extract target spatial information from sensing observations and construct position-aware CSI for the subsequent optimization stage. This enables the integration of target information acquisition and RIS-ISAC resource optimization.

- A deep optimization network is proposed to jointly generate RIS phase shifts and multi-user beamforming vectors based on the estimated spatial prior. By incorporating the reconstructed channel information, the proposed method improves sensing EE while considering the communication and sensing constraints.

- Extensive simulations are conducted to evaluate localization performance, sensing EE, computational efficiency, and the influence of important system parameters. The results verify the effectiveness of the proposed two-stage framework.

Comment 6: The results section lacks a qualitative comparison with existing studies and suggested to mention the ref number.

（结果部分缺乏与现有研究的定性比较，建议提及参考文献编号。）

【看看是不是能再找几篇比较新的文献来比较一下，再找个三篇左右】

Thank you for this suggestion. We have expanded the qualitative analysis in the simulation section and explicitly cited the corresponding references for the BCD-, SLBM-, and PF-based methods. The revised discussion now explains the observed performance differences in terms of iterative optimization, lower-bound approximation, fixed phase control, and the utilization of target-position information. Therefore, the results section now provides not only numerical comparisons but also a mechanism-based interpretation of why the proposed method performs differently from the existing approaches.

The performance improvement with increasing RIS elements is attributed to the enhanced spatial degrees of freedom provided by the RIS, which enables more flexible signal reflection control and beamforming optimization. However, the optimization based methods, including BCD [11], [34] and SLBM [37], rely on iterative updates of the RIS coefficients and beamforming vectors, and their performance depends on the optimization trajectory and initialization. The PF method [38] adopts a fixed phase control strategy and cannot fully exploit the available spatial information. In contrast, the proposed two-stage framework incorporates the estimated target position and reconstructed CSI as spatial priors, enabling the network to learn the nonlinear mapping from spatial-channel features to the joint RIS and beamforming configuration. Therefore, the proposed method can better exploit the additional spatial degrees of freedom introduced by increasing RIS elements.

Comment 7: The paper has several typos, for example in ref. [10] “ Spet.” Should be sep.

Thank you for pointing out these typographical errors. “Spet.” has been corrected to “Sep.” in Ref. [10]. We also identified and corrected the same error in Ref. [14]. In addition, we have carefully proofread the entire manuscript and corrected other minor typographical and formatting errors to improve the overall quality of the paper.

Reviewer: 2

Comment 1: Round-Trip Sensing Channel Formulation
The paper defines the equivalent channel vector from the BS to the target via the RIS as $b(\phi) = H^H \Theta h \in \mathbb{C}^{M \times 1}$, where $H \in \mathbb{C}^{N \times M}$ is the direct link between the BS and the RIS, and $h \in \mathbb{C}^{N \times 1}$ is the line-of-sight (LOS) link between the RIS and the target. The authors then define $B(\phi) = b(\phi)b(\phi)^H$ and express the received sensing signal-to-noise ratio (SNR) as $\text{SNR}r = \frac{\sum{k=1}^K \text{tr}(B(\phi) w_k w_k^H B^H(\phi))}{\sigma_r^2}$ (Eq. 14).  While mathematically this yields a scalar power value, the physical justification is flawed.
--> In a monostatic radar setup, the signal travels from the BS to the target and reflects back. Assuming channel reciprocity, the uplink channel from the target to the BS via the RIS should be the transpose of the downlink channel, i.e., $(h^T \Theta H)^T = H^T \Theta^T h$. The round-trip channel matrix for a point scatterer is typically modeled as $H_{rt} = H^T \Theta^T h h^T \Theta H$. The use of the Hermitian transpose $H^H$ for the downlink channel $b(\phi)$ and the subsequent construction of $B(\phi)$ using $b(\phi)b(\phi)^H$ assumes the uplink channel is the conjugate transpose of the downlink channel.
--> The authors must explicitly define the monostatic radar received signal model, clarify the round-trip channel matrix, and correct the SNR formulation to accurately reflect the physical propagation path.

（该论文将经由 RIS 从基站（BS）到目标的等效信道向量定义为 $b(\phi) = H^H \Theta h \in \mathbb{C}^{M \times 1}$，其中 $H \in \mathbb{C}^{N \times M}$ 表示 BS 与 RIS 之间的直连链路，$h \in \mathbb{C}^{N \times 1}$ 表示 RIS 与目标之间的视距（LOS）链路。随后，作者定义了 $B(\phi) = b(\phi)b(\phi)^H$，并将接收到的感知信噪比（SNR）表示为 $\text{SNR}_r = \frac{\sum_{k=1}^K \text{tr}(B(\phi) w_k w_k^H B^H(\phi))}{\sigma_r^2}$（式 14）。尽管从数学角度看这得出了一个标量功率值，但其物理依据存在缺陷。

--> 在单站雷达（monostatic radar）配置中，信号从 BS 传播至目标并反射回来。假设信道互易性，经由 RIS 从目标到 BS 的上行链路信道应为下行链路信道的转置，即 $(h^T \Theta H)^T = H^T \Theta^T h$。针对点散射体的往返信道矩阵通常建模为 $H_{rt} = H^T \Theta^T h h^T \Theta H$。在定义下行链路信道 $b(\phi)$ 时使用共轭转置 $H^H$，并随后利用 $b(\phi)b(\phi)^H$ 构建 $B(\phi)$，这种做法隐含了上行链路信道是下行链路信道的共轭转置这一假设。

--> 作者必须明确定义单站雷达的接收信号模型，阐明往返信道矩阵，并修正 SNR 公式，以准确反映物理传播路径。）

【GPT说和审稿人3的意见1是一致的】

Thank you very much for pointing out this important issue. We agree with the reviewer that the previous definition of the equivalent sensing channel vector b(φ)=HHΘhand the corresponding matrix B(φ)=b(φ)bH(φ)did not explicitly characterize the physical round-trip propagation process in a monostatic radar setup.

In the revised manuscript, we have corrected the RIS-assisted sensing channel formulation in Section II. Specifically, since the sensing signal propagates through the BS–RIS–target–RIS–BS path, we now define the forward BS–RIS–target channel as gf(φ)=hTΘH, and the reciprocal target–RIS–BS return channel as gr(φ)=HTΘTh.

Accordingly, the equivalent round-trip sensing channel matrix is rewritten as

B(φ)=αtgr(φ)gf(φ)=αtHTΘThhTΘH

where αtdenotes the complex target reflection coefficient, including the target radar cross section and reflection phase.

Based on this corrected round-trip sensing channel, the received radar echo signal is also revised as

yr(t)=B(φ)x(t-τ)+nr(t)

Then, the sensing SNR is calculated as

SNRr=k=1Ktr(B(φ)wkwkHBH(φ))σr2

It should be noted that the Hermitian transpose BH(φ)in the SNR expression is only used to compute the received echo power, while the round-trip propagation channel itself is now constructed using the transpose-based reciprocal link, i.e., HTΘTh. Therefore, the revised formulation explicitly captures the monostatic BS–RIS–target–RIS–BS propagation path and avoids treating the return channel as the conjugate transpose of the forward channel.

In addition, the corresponding descriptions in the sensing EE calculation and the EE-oriented loss function have been updated to ensure consistency with the corrected round-trip sensing channel model.

Comment 2: Line-of-Sight (LOS) Channel Matrix Typos

In equations (12) and (13), the LOS components $H_{LOS}$ and $h_{LOS}$ are defined using element-wise matrix representations. There are typographical errors in the exponential terms, specifically the inclusion of undefined variables such as $s_1 - s_2$ (e.g., $\rho_{Loss}^{B_m-R_n} e^{j \frac{2\pi}{\lambda} d \sin \vartheta_{n,m}}$ in the text, but rendered with extraneous characters in the equation). The authors should replace these cumbersome element-wise matrices with the standard, concise outer product of the array steering vectors, i.e., $H_{LOS} = \sqrt{N M} a_N(\theta_{AoA}) a_M(\theta_{AoD})^H$, which is standard in Q1 literature and eliminates ambiguity.

（在公式 (12) 和 (13) 中，视距（LOS）分量 $H_{LOS}$ 和 $h_{LOS}$ 是通过逐元素矩阵形式定义的。其指数项中存在排版错误，具体表现为包含了未定义的变量（例如 $s_1 - s_2$；文中表述为 $\rho_{Loss}^{B_m-R_n} e^{j \frac{2\pi}{\lambda} d \sin \vartheta_{n,m}}$，但在公式中却出现了多余字符）。作者应采用阵列导向矢量外积这一标准且简洁的形式来替代这些繁琐的逐元素矩阵表示，即 $H_{LOS} = \sqrt{N M} a_N(\theta_{AoA}) a_M(\theta_{AoD})^H$；这种表达方式在相关文献中是通用的，且能消除歧义。）

【关于公式12和公式13的修改和审稿人3的意见7相关】

公式12和13里面这个分子可能有问题，需要去检查一下原文的表述

We thank the reviewer for this valuable suggestion. The original element-wise expressions of HLoSand hLoShave been revised to improve clarity and eliminate the typographical ambiguity in the exponential terms. Since the proposed work adopts a position-based channel model in which the path loss and phase of each BS–RIS and RIS–target link are explicitly determined by their geometric locations, the LoS channel cannot be directly represented by the conventional steering-vector outer-product form NMaN(θAoA)aM(θAoD)H, which assumes uniform planar-wave propagation. Therefore, we retain the position-dependent channel formulation but rewrite it in a more concise matrix form instead of the previous element-wise expansion. The corresponding description has also been revised to improve readability.

The key revised content in the manuscript is as follows:

According to [33], HLoS∈CN×Mand hLoS∈CN×1can be compactly written by using the normalized ULA steering vectors as

HLoS=NMρLoss,BRaN(θAoA)aMH(θAoD),
hLoS=NρLoss,RVaN(θAoA),

where ρLoss,BR and ρLoss,RV denote the large-scale path loss coefficients of the BS–RIS link and the RIS–target link, respectively. θAoAand θAoDdenote the angle of arrival and the angle of departure of the corresponding LoS link, respectively. The normalized ULA steering vectors aN(⋅)and aM(⋅)are defined in (5) and (6).

Comment 3: Energy Efficiency Units

The sensing EE is defined as $\text{SNR}r / P{\text{total}}$ (Eq. 15). Since SNR is dimensionless and power is in Watts (Joules/second), the unit of EE is strictly $W^{-1}$ or $J^{-1}$. The authors state that the calculated EE is converted to a logarithmic scale and denoted as dB/J. While this is mathematically permissible (i.e., $10 \log_{10}(\text{EE})$), it is an unconventional unit for EE in communications literature. A brief explanatory note justifying this choice would improve clarity.

（感知能效（EE）定义为 $\text{SNR}r / P_{\text{total}}$（式 15）。由于信噪比（SNR）是无量纲量，而功率的单位为瓦特（焦耳/秒），因此 EE 的单位严格来说应为 $W^{-1}$ 或 $J^{-1}$。作者指出，计算出的 EE 被转换为对数标度，并记作 dB/J。尽管从数学角度看这是允许的（即 $10 \log_{10}(\text{EE})$），但这在通信文献中并非 EE 的常规单位。若能增加简短的说明来阐述采用该单位的理由，将有助于提升表述的清晰度。）

【单位问题和审稿人3的意见1一样】

Thank you for pointing out this important issue. We agree that since the sensing SNR is dimensionless and the denominator is the total power consumption measured in Watts, the linear unit of the sensing EE should be W-1. In the revised manuscript, we have clarified the unit of the linear sensing EE and the logarithmic representation used in the simulation results. Specifically, the sensing EE is defined as EE =SNRr/Ptotal, whose unit is W-1. For numerical presentation, the logarithmic sensing EE is defined as EE =10log10⁡(EEs/(1 W-1)), and it is reported in dB. Accordingly, the notation “dB/J” has been corrected to “dB” in the text and figure labels.

The key revised content in the manuscript is as follows:

Since SNRris dimensionless and Ptotalis measured in Watts, the linear unit of EE is W-1. For numerical presentation, we define the logarithmic sensing EE as EE =10log10⁡(EEs/(1 W-1)), which is reported in dB in the simulation results.

Comment 4: Hyperparameter Justification in the Loss Function

The comprehensive loss function (Eq. 18) introduces three weight coefficients ($\lambda_1, \lambda_2, \lambda_3$) to balance phase smoothness, power constraints, and user fairness. The authors state these are "determined through experimental tuning". However, this heuristic approach is a significant weakness. Deep learning models are highly sensitive to loss function weights. The authors should provide a systematic methodology for selecting these weights (e.g., grid search, Pareto front analysis, or dynamic weighting strategies like homoscedastic uncertainty) and include a sensitivity analysis demonstrating how variations in these parameters impact the final EE and constraint satisfaction.

（综合损失函数（式 18）引入了三个权重系数（$\lambda_1, \lambda_2, \lambda_3$），以平衡相位平滑性、功率约束和用户公平性。作者指出，这些参数是“通过实验调优确定的”。然而，这种启发式方法存在显著缺陷。深度学习模型对损失函数权重高度敏感。作者应提供一套系统性的权重选择方法（例如网格搜索、帕累托前沿分析，或同方差不确定性等动态加权策略），并进行敏感性分析，以展示这些参数的变化如何影响最终的能效（EE）及约束满足情况。）

Thank you for the constructive comment. We agree that simply stating that the loss-function weights are determined through experimental tuning is insufficient. In the revised manuscript, we have replaced the previous heuristic description with a two-step validation-based sensitivity analysis. Specifically, λ1, λ2, and λ3control the transmit-power penalty, communication SINR penalty, and sensing SNR penalty, respectively. Since the transmit power constraint can be directly evaluated from the beamforming outputs, λ1 is first determined independently through a one-dimensional validation search over {0,1,5,10,20}. Based on the logarithmic sensing EE and the power-violation magnitude of the unnormalized beamforming outputs, λ1=10 is selected.

Then, withλ1=10 fixed, λ2 and λ3 are jointly selected through a two-dimensional grid search over {1,5,10,20,40}. The logarithmic sensing EE, communication-SINR satisfaction rate, and sensing-SNR satisfaction rate are evaluated on the validation set. The added sensitivity results show that λ2=λ3=10 provides a balanced tradeoff between sensing-EE maximization and constraint satisfaction. Therefore, λ1=λ2=λ3=10 is adopted in the subsequent simulations.

To justify the selection of the loss-function weights, a two-step validation-based sensitivity analysis is conducted. In the comprehensive loss function, λ1, λ2, and λ3 control the transmit-power penalty, communication-SINR penalty, and sensing-SNR penalty, respectively. Since the transmit-power constraint can be directly evaluated from the beamforming outputs, λ1 is first determined independently before tuning the SINR- and SNR-related penalty weights. Specifically, λ1 is searched over {0, 1, 5, 10, 20} according to the logarithmic sensing EE and the power-violation magnitude of the unnormalized beamforming outputs. Based on this one-dimensional validation search, λ1 = 10 is selected and fixed in the subsequent twodimensional search.

With λ1 = 10 fixed, λ2 and λ3 are jointly searched over {1, 5, 10, 20, 40}. For each weight combination, the EE optimization network is retrained under the same network architecture, dataset partition, and training strategy, while Loc-Net is kept fixed. The trained model is evaluated on the validation set in terms of logarithmic sensing EE, communication-SINR satisfaction rate, and sensing-SNR satisfaction rate.

| (a) | (b) | (c) |
| --- | --- | --- |

Fig. 3. Sensitivity analysis of loss penalty weights. (a) sensing EE. (b) SINR satisfaction rate. (c) SNR satisfaction rate.

As shown in Fig. 3, relatively small values of λ2 and λ3 allow the network to prioritize sensing EE maximization but result in lower communication SINR and sensing SNR satisfaction rates. Increasing λ2 mainly improves the communication-SINR satisfaction rate, whereas increasing λ3 mainly improves the sensing-SNR satisfaction rate. However, excessively large penalty weights make the loss dominated by the constraint terms and degrade the sensing EE. Therefore, λ2 = λ3 = 10 is selected as a balanced setting between sensing-EE maximization and constraint satisfaction. Together with the independently selected λ1 = 10, all three penalty weights are set to 10 in the subsequent simulations.

Comment 5: Fairness of Baseline Comparisons

The simulation section compares the proposed DL method against traditional optimization algorithms: Block Coordinate Descent (BCD), Successive Lower Bound Maximization (SLBM), and Phase Fixing (PF). It is ambiguous whether these traditional baselines are executed using the perfect CSI or the imperfect/estimated CSI derived from the first stage of the proposed method. If the BCD algorithm is provided with perfect CSI while the DL method uses estimated CSI, the comparison is fundamentally unfair. Conversely, if BCD is forced to use the imperfect CSI, its performance degradation should be explicitly attributed to the CSI error. The authors must clearly state the CSI assumptions for all baselines to ensure a rigorous and fair comparative analysis.

（仿真部分将所提出的深度学习（DL）方法与几种传统优化算法进行了比较，这些算法包括：块坐标下降（BCD）、连续下界最大化（SLBM）和相位固定（PF）。关于这些传统基准算法是基于理想信道状态信息（CSI）还是基于所提方法第一阶段得出的非理想（即估计）CSI 进行运行，文中表述尚不明确。如果 BCD 算法使用理想 CSI 而 DL 方法使用估计 CSI，这种比较在本质上是不公平的；反之，若强制 BCD 使用非理想 CSI，则其性能下降应明确归因于 CSI 误差。作者必须明确说明所有基准算法所采用的 CSI 假设，以确保比较分析的严谨性与公平性。）

【说明对比试验方法里面的CSI是怎么获取的，明确公平性】

Thank you for pointing out this important issue. We agree that the CSI assumption for the benchmark methods should be explicitly clarified to ensure a fair comparison. In the revised manuscript, we have added a clear description of the CSI assumptions used by all compared methods. Specifically, under the unknown-target setting, the BCD-based, SLBM-based, PF-based, and proposed DL methods are all executed using the same estimated position-aware CSI reconstructed from the first-stage Loc-Net. No benchmark method is provided with perfect target-related CSI in the main comparison. After each method generates the RIS phase shifts and BS beamforming vectors, the sensing EE, communication SINR, and sensing SNR are recalculated over the same test channels. Therefore, the comparison is conducted under the same estimated-CSI condition. The perfect-CSI/location case is only used as an upper-bound reference in the ablation study and is not used in the main comparison with BCD, SLBM, and PF.

The key revised content in the manuscript is as follows:

For a fair comparison under the unknown-target setting, all algorithms are evaluated under the same estimated-CSI condition. Specifically, the BCD-based, SLBM-based, and PF-based methods use the position-aware CSI reconstructed from the first-stage Loc-Net, rather than perfect target-related CSI. The proposed DL method also uses the same estimated CSI for the second-stage EE optimization. After obtaining the RIS phase shifts and BS beamforming vectors, all methods are evaluated over the same test channels. The perfect-CSI/location case is only used as an upper-bound reference in the ablation study and is not used in the main comparison with BCD, SLBM, and PF.

Comment 6: Dataset Generalization and Robustness

The DL model is trained on a dataset generated via MATLAB for a specific Rician channel model with a fixed Rician factor ($K=5$) and specific array geometries. The manuscript lacks a discussion on the model's generalization capabilities. The authors should evaluate the trained model's robustness by testing it against channel conditions not seen during training, such as varying Rician factors, different numbers of users, or slight perturbations in the array element spacing.

（该深度学习模型是基于特定莱斯（Rician）信道模型（具有固定的莱斯因子 $K=5$ 和特定的阵列几何结构）生成的数据集进行训练的。文中缺乏对该模型泛化能力的讨论。作者应通过在训练过程中未涉及的信道条件下（例如变化的莱斯因子、不同的用户数量或阵列阵元间距的微小扰动）进行测试，来评估已训练模型的鲁棒性。）

【补实验，不同莱斯因子，不同用户数量下的能效表现，看鲁棒性】

Thank you for the constructive comment. We agree that the generalization capability of the trained DL model should be further evaluated. In the revised manuscript, we have added a robustness evaluation under unseen channel conditions. Specifically, the model is trained under the nominal setting with KR=5and d=λ/2, and is then directly tested without retraining or fine-tuning. Two types of unseen channel variations are considered: different Rician factors KR∈{1,3,5,7,10}and array-spacing perturbations d=(1+ϵ)λ/2, where ϵ∈{-10%,-5%,0,+5%,+10%}. For each setting, an independent test set with 2,000 samples is generated. The logarithmic sensing EE is reported to evaluate the performance robustness under these unseen channel conditions. The added results show that the proposed method maintains stable performance, which demonstrates its robustness to channel-distribution and array-geometry variations.

The key revised content in the manuscript is as follows:

To evaluate the generalization capability of the proposed model, we further test the trained network under unseen channel conditions without retraining. The model is trained under the nominal setting with KR=5and d=λ/2. During testing, the Rician factor is varied as KR∈{1,3,5,7,10}, and the array spacing is perturbed as d=(1+ϵ)λ/2, where ϵ∈{-10%,-5%,0,+5%,+10%}. For each setting, an independent test set with 2,000 samples is generated, while the other system parameters remain unchanged.

| (a) Rician Factor Variation | (b) Array Spacing Perturbation |
| --- | --- |

Fig. 13. Model robustness analysis.

As shown in Fig. X(a), the logarithmic sensing EE slightly decreases when KRbecomes smaller, since a smaller KRindicates a stronger NLoS component and higher channel randomness. When KRis larger than the nominal setting, the sensing EE remains stable, indicating that the proposed model can generalize to different LoS/NLoS power ratios. Fig. X(b) shows that the sensing EE reaches its best performance near the nominal spacing d=λ/2, and slightly decreases as the spacing perturbation increases. This is because the perturbation changes the phase progression of the ULA steering vectors and introduces array-geometry mismatch. Nevertheless, the degradation remains limited under both types of unseen variations, which verifies the robustness of the proposed model to channel-distribution and array-geometry variations.

Comment 7: Discuss recent advancements in unsupervised and model-driven learning for ISAC beamforming. For instance, comparing their residual learning approach to deep unfolding techniques (which unroll iterative optimization algorithms into neural networks) would provide a stronger theoretical grounding.

（探讨面向 ISAC（通信感知一体化）波束赋形的无监督学习与模型驱动学习领域的最新进展。例如，将这些方法中的残差学习策略与深度展开技术（即将迭代优化算法展开为神经网络）进行对比，将有助于建立更坚实的理论基础。）

【文字表述性的，看最新的文献，ISAC波束成型的无监督学习模型与模型驱动学习领域的最新进展】

Thank you for this valuable suggestion. In the revised manuscript, we have expanded the literature review to include recent advances in unsupervised learning, model-driven learning, and deep unfolding for ISAC waveform and beamforming design. Representative studies published in recent years have been added. We have also briefly clarified the methodological distinction between residual learning and deep unfolding to provide a clearer theoretical context for the residual learning strategy adopted in this work. The corresponding revision has been added to the Introduction as follows:

Recent studies have further explored unsupervised and model-driven learning for ISAC waveform and beamforming design. An unsupervised learning-based framework was developed in [24], where the sensing–communication tradeoff and system constraints were incorporated into the network training process without requiring labeled optimal waveforms. Deep unfolding has also been introduced into RIS-aided ISAC by mapping iterative optimization steps into trainable network layers [25]. Furthermore, a model-driven learning approach was developed in [26] by unfolding an ADMM-based alternating optimization procedure for joint waveform and beamforming design. Unlike deep unfolding, which explicitly maps the iterations of a prescribed optimization algorithm into network layers, residual learning refines an initial output by learning its residual correction.

Comment 8: Acknowledge recent works on multi-task learning specifically applied to RIS channel estimation and localization, highlighting how their specific multi-view feature fusion architecture offers superior performance or lower overhead compared to contemporary DL-based estimators.

（提及近期应用于RIS信道估计与定位的多任务学习研究，并强调其特定的多视图特征融合架构相比现有的基于深度学习（DL）的估计器，具有更优的性能或更低的开销。）

【表述性的，按照他说的在文章中补上最新的这块领域的知识，可以引用个两篇这块领域的文献】

We appreciate the reviewer’s helpful suggestion. In the revised manuscript, we have added recent studies on multi-task learning-based RIS channel estimation and joint RIS channel estimation and localization. We have also further explained the multi-view feature fusion and shared multi-task structure adopted in Loc-Net. In particular, the revised discussion clarifies that the shared feature extractor processes multiple channel representations and supports the related localization tasks without constructing separate feature extraction networks. The corresponding revision has been added to the Introduction as follows:

Recent studies have also investigated multi-task and joint learning-based designs for RIS channel estimation and localization. In [17], a multi-task network was developed to exploit the channel correlations among different users and STAR-RIS elements for near- and far-field cascaded channel estimation with limited pilot overhead. More recently, a DNN-assisted framework was proposed in [18] to jointly address channel estimation and localization in RIS-assisted mmWave MISO systems. In the proposed Loc-Net, the real/imaginary, amplitude/phase, and position-encoding features are processed through multi-view feature fusion, while position classification and distance-angle regression share a common feature extractor. Compared with constructing separate networks for the related localization subtasks, the shared feature extractor avoids duplicated feature extraction and network parameters.

Reviewer: 3

Comment 1: The sensing metric in (14) sums the contributions of the communication beamformers w_k, although Section II-A states that w_k = 0 in the dedicated sensing phase, and it omits the fading coefficient beta, the path loss, and the target radar cross section that appear in (9). Please make the definition consistent with the transmission model, and correct the unit of the energy efficiency, which is reported as dB/J although the denominator is a power.

（式 (14) 中的感知度量指标累加了通信波束赋形向量 $w_k$ 的贡献，尽管第 II-A 节指出在专用感知阶段 $w_k = 0$；此外，该式还遗漏了式 (9) 中出现的衰落系数 $\beta$、路径损耗以及目标雷达散射截面。请确保该定义与传输模型保持一致，并修正能效的单位——目前的标注为 dB/J，但其分母项实为功率。）

【单位问题和审稿人2的意见3一样】

Thank you for the valuable comment. We have revised the sensing SNR definition and the EE unit to make them consistent with the transmission model. First, we clarified that the sensing EE optimization in this paper is conducted in the ISAC dual-function phase. In this phase, no dedicated radar waveform is transmitted, and the communication beamformers {wk}are reused for target sensing. Therefore, the sensing SNR in Eq. (14) is calculated by summing the contributions of the communication beamformers. The statement related to wk=0in the sensing phase has been revised to avoid ambiguity.

Second, we clarified that the large-scale path loss and small-scale fading are included in the BS–RIS channel H and the RIS–target channel h, while the target radar cross section is included in the reflection coefficient αt=σRCSejψt. Therefore, the path loss, fading coefficient, and target RCS are incorporated into the round-trip sensing channel B(φ) and hence into the sensing SNR expression.

Finally, we corrected the unit of the sensing EE. The linear sensing EE has the unit W-1, and the logarithmic value is now reported in dB with respect to 1 W-1, instead of “dB/J”.

The key revised content in the manuscript is as follows:

Specifically, the large-scale path loss and small-scale fading are included in Hand h, while the target RCS is included in the reflection coefficient αt. Therefore, the path loss, fading effect, and target RCS are incorporated into the round-trip sensing channel B(φ). By substituting (10) and (11) into the round-trip sensing channel definition, the equivalent channel matrix B(φ)can be obtained. In the ISAC dual-function phase considered for sensing EE optimization, the communication beamformers are reused for target sensing and no dedicated radar waveform is transmitted. Therefore, the received sensing SNR is expressed as

SNRr=k=1Ktr(B(φ)wkwkHBH(φ))σr2         (14)

The key revised content regarding the EE unit in the manuscript is as follows:

Since SNRr is dimensionless and Ptotal is measured in Watts, the linear unit of EE  is W-1. For numerical presentation, we define the logarithmic sensing EE as EE =10log10⁡(EEs/(1 W-1)), which is reported in dB in the simulation results.

Comment 2: Section II states that there is no cascaded channel between the RIS and the UEs, so the RIS phases do not affect the SINR in (4). The constraint (16d) is nevertheless treated as depending on the RIS configuration throughout Sections II-C and IV. Please clarify the role of the RIS in the communication link, or remove the dependence.

（第二部分指出 RIS 与用户设备（UE）之间不存在级联信道，因此 RIS 相位不影响式 (4) 中的 SINR。然而，在第二部分 C 节和第四部分中，约束 (16d) 仍被视为依赖于 RIS 配置。请明确 RIS 在通信链路中的作用，或消除这种依赖关系。）

【大概是说明RIS和UE之间是没有直接信道的，RIS是和target有联系的。这个要好好看下，到底咋回事，反正最好是不要改系统模型和后面的仿真啥的】

Thank you for this valuable comment. We agree that, in the considered system model, there is no RIS–UE cascaded channel. The RIS is deployed to establish and adjust the BS–RIS–target sensing path, while the communication UEs are served through the direct BS–UE links. Therefore, the RIS phase shifts do not directly affect the UE SINR in (4).

In the revised manuscript, we have clarified the role of the RIS in the communication and sensing links. Specifically, the SINR constraint in (16d) is now described as a communication QoS constraint on the BS beamforming vectors, rather than a RIS-dependent constraint. This constraint is retained because the same beamforming vectors are reused for both downlink communication and target sensing in the ISAC dual-function phase. Hence, although the RIS phase matrix does not directly affect the UE SINR, the communication SINR constraint still restricts the feasible beamforming vectors and indirectly affects the sensing EE optimization.

We have revised the related descriptions in Sections II-A, II-C, III-B, and IV-A to remove the possible ambiguity and to explicitly state that all SINR-related evaluations are calculated using the direct BS–UE channel without assuming any RIS–UE cascaded channel.

Section II-A

It should be noted that, in the considered system geometry, the RIS is used to establish the BS–RIS–target sensing path and there is no RIS–UE cascaded channel. Therefore, the UE SINR in (4) is determined only by the direct BS–UE channel hd,kand the BS beamforming vectors {wj}, while it is independent of the RIS phase matrix Θ.

Section II-C

From (4), the communication SINR depends on the direct BS–UE channel hd,kand the beamforming vectors {wj}, but it is independent of the RIS phase matrix Θ. In contrast, the sensing SNR and sensing EE depend on the RIS-assisted round-trip sensing channel B(ϕ), which is determined by Θ, H, and h. Therefore, in problem (16), Θis optimized to enhance the sensing link, while {wk}is jointly optimized to balance sensing EE maximization and direct-link communication QoS.

These SINR constraints do not directly involve the RIS phase matrix, but they restrict the feasible beamforming vectors. Since the same beamforming vectors are reused for target sensing, the communication constraints still affect the sensing EE optimization indirectly through {wk}.

Section III-B

Meanwhile, to guarantee the communication QoS of UEs, an SINR penalty term LSINRis added according to (4). This term is computed from the direct BS–UE channel and the beamforming vectors, and does not introduce any RIS–UE cascaded channel.

Section IV-A

In all SINR-related evaluations, the SINR is calculated using the direct BS–UE channel according to (4). The RIS phase shifts are not assumed to affect the UE SINR. The SINR satisfaction rate is reported to show whether the optimized beamforming vectors satisfy the communication QoS constraints while maximizing the sensing EE.

Comment 3: The echo model in (9) is built from LoS steering vectors only, whereas the channels used for the energy efficiency computation in (10) and (11) are Rician with a NLoS component. Please use the same channel model in both, and define the dimension N_t of the received echo vector.

（式 (9) 中的回波模型仅基于视距（LoS）导向矢量构建，而式 (10) 和 (11) 中用于能效计算的信道则为包含非视距（NLoS）分量的莱斯（Rician）信道。请在两者中采用相同的信道模型，并定义接收回波向量的维度 $N_t$。）

【关于模型定义的，要谨慎，不能影响整体的系统模型和后面的仿真】

Thank you very much for pointing out this inconsistency. We agree that the previous echo model in Eq. (9) was built from the LoS steering-vector representation, while the channels used for the sensing SNR and sensing EE calculation were modeled as Rician channels with both LoS and NLoS components. This could lead to an inconsistency between the radar echo model and the EE calculation. We also agree that the dimension Ntof the received echo vector was not clearly defined.

To address this issue, we have removed the previous LoS steering-vector-based echo model and rewritten the received echo signal using the same RIS-assisted round-trip sensing channel used for the sensing SNR and EE calculation. This correction is also consistent with the round-trip sensing channel revision made in response to Reviewer 2, Comment 1. Specifically, the received echo signal is now modeled based on the round-trip channel B(φ), which is constructed from the BS–RIS channel H and the RIS–target channel h. Since H and h are modeled as Rician channels with both LoS and NLoS components, the echo signal model, sensing SNR, and sensing EE calculation are now based on the same channel model.

The key revised content in the manuscript is as follows:

Based on the RIS-assisted monostatic propagation path, the received echo signal at the BS is modeled as

yrt=Bφxt-τ+nrt           (9)

where yr(t)∈CM×1is the echo signal received by the M-antenna BS, τ=2(r1+r2)/cis the total round-trip delay, and r1and r2denote the BS–RIS distance and the RIS–target distance, respectively. nr(t)∈CM×1denotes the complex additive white Gaussian noise with zero mean and covariance σr2IM. Doppler shift is not considered in this paper. The BS–RIS channel Hand the RIS–target channel hused in B(φ)are specified as follows.

We further added the following clarification after the Rician channel definitions:

The channels Hand hin B(φ)follow the above Rician model. Therefore, the received echo signal, the sensing SNR, and the sensing EE are constructed from the same BS–RIS and RIS–target channel realizations, which contain both LoS and NLoS components.

Moreover, the sensing SNR is calculated using the same equivalent sensing channel matrix as

SNRr=k=1K tr⁡(B(φ)wkwkHBH(φ))σr2.
Therefore, the revised formulation removes the inconsistency between the LoS-only echo model and the Rician-channel-based EE calculation. The dimension of the received echo vector has also been corrected from CNt×1to CM×1, since the echo signal is received by the M-antenna BS, and the undefined notation Nthas been removed.

Comment 4: The second-stage network enforces the unit-modulus, power, SINR, and sensing SNR constraints only through penalty terms in the loss function. Please report the constraint violation statistics on the test set, since the reported energy efficiency is not attainable if (16c)-(16e) are violated.

（第二阶段网络仅通过损失函数中的惩罚项来强制满足单位模、功率、SINR 及感知 SNR 约束。请报告测试集上的约束违反统计数据，因为一旦违反 (16c)–(16e) 中的约束，所报告的能效便无法实现。）【和审稿人1的审稿意见4类似】

We agree that penalty-based training alone cannot guarantee that every output is feasible. In the revised manuscript, the RIS unit-modulus constraint is enforced through phase-to-complex-coefficient mapping, and the transmit-power constraint is enforced through beamforming normalization. A sensing-SNR penalty term has also been added to the loss function. Furthermore, we report the power violation, communication-SINR satisfaction rate, sensing-SNR satisfaction rate, and joint feasibility rate on the 2,000-sample independent test set. The sensing EE calculated only over jointly feasible samples is also reported to verify that the EE performance is not obtained through constraint violations.

The key revised content in the manuscript is as follows:

To verify the feasibility of the proposed network outputs, the power, communication-SINR, and sensing-SNR constraints are evaluated on the independent test set with 2,000 samples. For each sample, the total transmit power, user SINRs, and sensing SNR are recalculated using the corresponding test channels. A sample is regarded as feasible only when constraints (16c)–(16e) are simultaneously satisfied. The power, communication-SINR, and sensing-SNR satisfaction rates are defined as the percentages of test samples satisfying k=1K ∥wk∥22≤Pmax, SINRk≥γk, and SNRr≥τr, respectively. The joint feasibility rate and the average sensing EE over feasible samples are also reported.

Table X Constraint Satisfaction Statistics

| Metric | result |
| --- | --- |
| Power-constraint satisfaction rate | 100% |
| Communication-SINR satisfaction rate | 98.5% |
| Sensing-SNR satisfaction rate | 99.1% |
| Joint feasibility rate | 97.75% |
| EE over all test samples | 81.97dB |
| EE over feasible test samples | 81.75dB |

As shown in Table X, the transmit-power constraint is satisfied for all test samples because the generated beamforming vectors are normalized according to Pmax. The communication-SINR and sensing-SNR constraint satisfaction rates are 98.50% and 99.10%, respectively. Consequently, 97.75% of the test samples simultaneously satisfy constraints (16c)–(16e). Moreover, the average sensing EE calculated over the feasible samples is 81.72, which is close to 81.85 obtained over all test samples. These results demonstrate that the sensing-EE performance of the proposed method is not achieved by systematically violating the power, communication, or sensing constraints.

Comment 5: The green benefit is claimed while the energy consumed by the two networks is excluded from the power model, with the statement in Section II-B that the deep learning approach "maintains P0 at a minimal level". Please include the inference cost of both networks in the total power and report the offline training cost, so that the comparison with the iterative baselines is fair.

（在声称具有“绿色”效益时，相关论述（见第二部分B节）指出该深度学习方法能“将 P0 维持在最低水平”，但却将这两个网络所消耗的能量排除在了功耗模型之外。请将这两个网络的推理开销计入总功耗，并列出离线训练开销，以确保与迭代式基准方法之间的比较公平合理。）

【将两个网络推理的开销计入总功耗，这个是否合理？（这个总功耗怎么来计算呢？怎么确定呢？）其次是加入总功耗的话那么整个EE不就都会改变了吗，因为P是分母啊，这个问题要慎重考虑。如果只单纯列出训练开销可能会好一些。】

Thank you for this valuable comment. According to the reviewer’s suggestion, we have revised the power consumption model to explicitly include the online inference cost of the proposed two-stage DL model. Specifically, the total power consumption now includes an additional term Pinf, which accounts for the inference power of both Loc-Net and ResidualEnergyNet. Accordingly, the sensing EE formulation and the related EE results have been updated based on the revised power model.

We have also added the estimation of the offline training cost in the revised manuscript. Since the training process is performed only once before deployment, the offline training cost is reported separately and is not repeatedly included in the per-slot sensing EE calculation.

where Ptotal=1ηk=1K ∥wk∥22+P0+Pinfis the total power consumption. P0denotes the static circuit and baseband processing power, and Pinfdenotes the online inference power consumed by the proposed two-stage DL model. Specifically, Pinfis calculated as

Pinf=PGPUtinfTupd

where PGPUis the average GPU power during inference, Tupdis the system configuration update period, and tinf=tloc+toptis the total inference time of Loc-Net and ResidualEnergyNet. Since SNRris dimensionless and Ptotalis measured in Watts, the linear unit of EEsis W-1. For numerical presentation, the logarithmic sensing EE is defined as EEs,dB=10log⁡10(EEs/(1W-1)), which is reported in dB in the simulation results. Compared with traditional iterative optimization, the proposed DL method reduces the online solving complexity by replacing repeated iterations with the two forward propagations of Loc-Net and ResidualEnergyNet, whose inference power is explicitly included in Pinf.

The offline training cost of the proposed two-stage DL method is also reported. Following the common energy-estimation practice in energy-aware deep learning, the training energy is estimated as

Etrain=PtrainTtrain

where Ptrainis the average GPU power during training and Ttrainis the total offline training time. For the default setting with N=32, the average training time per epoch is 34.59 s, and the two-stage DL model is trained for 100 epochs. Therefore, the total offline training time is Ttrain=100×34.59=3459s, i.e., approximately 0.96 h. Using an average GPU training power of 70 W on the RTX 4060 GPU, the estimated offline training energy is Etrain≈2.42×105J, or 0.067 kWh. Since training is performed only once before deployment, this offline cost is reported separately and is not repeatedly included in the per-slot sensing EE.

Comment 6: In Section V the text reports a maximum energy efficiency of 89.74 dB/J decreasing to 83.62 dB/J, while the caption of Fig. 7 reports the range 83.52-88.74 dB/J. Please correct these values, and explain what is held fixed in Fig. 7(b), where the communication rate decreases as the SNR increases.

（文中第五部分指出最大能效从 89.74 dB/J 降至 83.62 dB/J，而图 7 的图注则显示该范围为 83.52–88.74 dB/J。请更正这些数值，并说明在图 7(b) 中保持不变的参数是什么——在该图中，通信速率随信噪比（SNR）的增加而降低。）

【实验结果图上数值和文字表述要对应，并且说明在实验中不变的参数是什么？总体上是文字表述性的问题】

Thank you for pointing out this inconsistency and the need for clarification. We have corrected the mismatch between the numerical values reported in the text and the figure caption. We have also revised the description of Fig. 8 to clarify that the horizontal axis corresponds to the sensing-SNR threshold τr, rather than the communication-channel SNR.

In this experiment, only τris varied, while the other system parameters are kept fixed, including the number of BS antennas M, the number of RIS elements N, the number of UEs K, the transmit-power budget, the communication-SINR requirement, and the channel generation settings. The decrease in communication rate is explained by the stricter sensing-SNR requirement: since the communication beamforming vectors are reused for target sensing in the ISAC dual-function phase, increasing τrmakes the beamforming design more sensing-oriented under the fixed transmit-power budget, which may reduce the achievable communication rate.

To study the impact of the sensing-SNR requirement on system performance, the sensing-SNR threshold τrin (16e) is varied from 5 to 25 dB. During this experiment, only τris varied, while the other parameters are kept fixed, including the number of BS antennas M=8, the number of RIS reflecting elements N=32, the number of UEs K=2, the transmit-power budget, the communication-SINR threshold, and the channel generation settings.

Comment 7: Please correct the following: the power constraint (16c) omits the dedicated radar term Tr(W0) that appears in (2); the sentence "the matrixes H and h cannot be obtained due to unknown d and theta_r" uses symbols not defined in (12) and (13); and the typographical errors "assmued", "appliing", and "By appliing similar optimization logic".

（请更正以下内容：功率约束 (16c) 遗漏了 (2) 式中出现的专用雷达项 $\text{Tr}(\mathbf{W}_0)$；

关于“由于 $d$ 和 $\theta_r$ 未知，无法获得矩阵 $\mathbf{H}$ 和 $\mathbf{h}$”的表述中，使用了 (12) 和 (13) 式未定义的符号；

以及拼写错误：“assmued”、“appliing”和“By appliing similar optimization logic”。）

【和审稿人2的意见2相关

整体分成了三个问题，一个是功耗遗漏，但是谈到功耗的话那就会和意见5里面的两阶段网络推理功耗相关联了，这是个棘手的问题。一个是公式12和公式13定义的问题，这个在审稿人2中做修改。还有一个是拼写相关的问题，比较简单一些。】

We thank the reviewer for carefully pointing out three types of defects: the missing radar beam term in the power constraint (16c), mismatched undefined symbols d and θr in the text that conflict with original Eqs. (12) and (13), and multiple spelling typos. The symbol inconsistency issue originates from the unstandardized channel formula in original Eqs. (12)-(13), which we jointly revised together with Comment 2 by reconstructing the LoS channel matrix and standardizing all geometric parameters. We have supplemented the missing constraint item, rewritten the inconsistent sentence to match the updated symbol system, and fully corrected all spelling errors across the full text.

The key revised content in the manuscript is as follows:

k=1K ‖wk‖22+Tr⁡(W0)≤Pmax         16(c)

However, in this work, the target location is unknown. Therefore, the target-related geometric parameters, such as the RIS–target distance and angle, cannot be directly obtained. As a result, the RIS–target channel hand the corresponding round-trip sensing channel

B(φ)=αtHTΘThhTΘH

are unknown, which makes the sensing EE maximization problem more challenging.

This correction is also consistent with the compact LoS channel representation revised in response to Reviewer 2, Comment 2, where the original element-wise LoS matrices have been replaced by steering-vector-based expressions and the related steering vectors have been clearly specified.